|  | STANDARDS ASSOCIATION OF ZIMBABWE |
| --- | --- |

DRAFT FOR PUBLIC COMMENT

LATEST DATE FOR RECEIPT OF COMMENTS: 02-12-2024

Our ref: – BC 012– D1020/1 Draft Number: BC – D1020/1-Part 1 Date: 16-10-2024

TECHNICAL COMMITTEE   BC 012: BIOGAS SYSTEMS

PART 1: DESIGN, INSTALLATION, OPERATION AND MAINTENANCE OF BIOGAS SYSTEMS

DRAFT ZIMBABWE STANDARD FOR REQUIREMENTS FOR BIOGAS SYSTEMS PART 1: DESIGN, INSTALLATION, OPERATION AND MAINTENANCE OF BIOGAS SYSTEMS

This draft is now available for public comment.  Your views and technical comments on it would be appreciated.  If you have no specific comments to make but find it generally acceptable it would be helpful if you would notify us accordingly.  Suggestions entailing revisions of the text should indicate the preferred wording using the attached template.  The relevant clause number should be quoted against any comment.

All comments should be sent to the Committee Secretary Mr P Chindara at the address shown below.

Standards Association of Zimbabwe

P O Box 2259

Harare

E-mail: pchindara@saz.org.zw

Website:  www.saz.org.zw

THIS IS A DRAFT AND MUST NOT BE REGARDED OR USED AS A ZIMBABWE STANDARD.

Copyright Users of Zimbabwe National Standards are reminded that copyright subsists in all ZWS publications. No part of this publication may be produced in any form or by any means, electronic or mechanical, including photocopying or posting on the internet or an intranet, without permission in writing from the Director General of the Standards Association of Zimbabwe. Orders for all national, international and foreign standards publications should be addressed to: Director – Standards Development, Information and Training Standards Association of Zimbabwe P O Box 2259 Harare Northridge Park Borrowdale Tel: 885511-2, 882017-9, 882021-2 Fax: 882020 E-mail: info@saz.org.zw Website: www.saz.org.zw

It is important that users of Zimbabwe Standards should ensure that they are in possession of the latest amendments or editions.

AMENDMENTS

| No. | MD No. | Date of Issue | Text affected |
| --- | --- | --- | --- |

Zimbabwe Standards are revised, when necessary, by issuing either amendments or revised editions.  Suggestions for improvements will be welcome at all times.

- NOTICE

The Standards Association of Zimbabwe is the registered owner of certification and registration marks shown in Information Services Publication No. 2, one of which is shown below.  The presence of a certification mark on a product is an independent assurance that the product complies with the requirements of the relevant Zimbabwe Standard.

A certification mark may be used by the manufacturers only under licence from the Association.  Particulars of the conditions under which licences are granted may be obtained from:

Director General

Standards Association of Zimbabwe

P O Box 2259

Harare

Northridge Park

Borrowdale

Telephone: 885511-2/882017-9/882021

Fax: 882020

E-mail:     info@saz.org.zw

Website:  www.saz.org.zw

Contents										Page

Foreword 		vi

Introduction 		4

- Scope 		5

2. 	Definitions		5

- Biogas plant construction 		6

4. 	Plant construction		14

5.	Piping system 		19

6.	Testing newly constructed biogas systems		23

7	operation and maintenance of biogas systems		24

8.	Environmental safeguards		26

9.	Warranty 		26

10.	Decommissioning of a biogas system		26

Appendix A – frictional coefficients of selected

materials		27

Appendix B – Biogas Plant Construction Procedure		28

Foreword

This Zimbabwe Code of Practice ZWS  1020: Biogas systems: Part 1:2024: Design, installation, operation and maintenance of biogas systems, is based on Zambian Standard ZS 908: Part 1: 2015.and ISO 20675:2018

The standard was prepared by technical committee BC 012: Bioenergy and Biogas, under the general guidance of the Building and Civil Engineering sector.

This standard refers to the following publications:

ZWS ISO 23590	: 	Household biogas system requirements: design, installation,

operation, maintenance and safety

ZWS ISO 20675 	:  	Biogas - Biogas production, conditioning, upgrading and

utilization — Terms, definitions and classification scheme

ZWS EN 197 		: 	Cement

Part 1:   	 		Composition, specifications and conformity criteria for common

cement

ZWS EN 197		:  	Cement

Part 2:   			Conformity evaluation

ZWS ENV 413	:  	Masonry cement

Part 1:   			Specification

ZWS 1020 		:	Code of practice for biogas systems

Part 2		: 	Micro-grids.

Introduction

Biogas is a clean combustible gas produced from the anaerobic digestion of biodegradable organic material by micro-organism under a temperature range of 10 to 60 degree Celsius, pH(6.8-7.5) and moisture content(. The feedstock that can be used for production of biogas include animal waste, agricultural waste, plant material, sewage, or food waste. Biogas is typically made up of 50% to 70% methane, 30% to 50% carbon dioxide. Other gases such as hydrogen sulphide, ammonia and water vapour can be present in lower proportions.  Traces of hydrogen, oxygen and nitrogen gases can also be found. It is about 20% lighter than air with an ignition temperature of 6500 to 7500 oC, has a calorific value of 20MJ/m3 and its burning and lighting efficiencies are approximately 60% and 2% respectively. Biogas can be used for all applications designed for gaseous and liquid fuels and this includes the following;

- Heating

- Cooking

- Refrigeration

- Cooling

-	Lighting

-	Generation of electricity

-	Fuel for combustion engines

However, biogas requires further cleaning and upgrading if it is to be used as vehicle fuel.

A variety of factors affect the rate of digestion and biogas production. The most important are type of feedstock, moisture content or mixing ratio, loading rate, pH level, carbon to nitrogen ratio, toxicity and retention time. Anaerobic bacteria communities can endure temperatures ranging from below freezing to above 57.2°C, but they thrive best at temperatures of about 36.7°C (mesophilic) and 54.4°C (thermophilic). To have an optimal digestion process, digesters must be kept at a consistent temperature, as rapid changes will interrupt bacterial activity.

ZIMBABWE CODE OF PRACTICE

FOR

BIOGAS SYSTEMS

PART 1: DESIGN, INSTALLATION, OPERATION AND

MAINTENANCE OF BIOGAS SYSTEMS

- Scope

This Zimbabwean Code of practice provides requirements for design, installation operation and maintenance of biogas digester systems. This standard shall be read in conjunction with ZWS 1020: Part 2.

The following topics are excluded from this document:

- Boilers, burners, furnaces and lightening, in case these are not specifically applied for locally produced biogas;

- Gas-fuelled engines for vehicles and ships;

- The public gas grid;

- specifications to determine biomethane quality;

- Transportation of compressed or liquefied biogas;

- Transportation of biomass or digestate;

- Assessment and determination whether biomass is sourced sustainably or not.